## 01. IDENTIFICATION OF THE SUBSTANCE/PREPARATION & THE COMPANY/UNDERTAKING

| 1.1 | Product Identifier |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name |  |  | Mandarin Oil. |  |  |  |  |
| Biological Definition |  |  | Citrus Nobilis Peel Oil Expressed is an essential oil expressed from the epicarps of the Mandarin, Citrus nobilis, Rutaceae. |  |  |  |  |
| INCI Name |  |  | Citrus Nobilis Peel Oil Expressed |  |  |  |  |
| Synonyms & Trade Names |  |  | - |  |  |  |  |
| CAS-No |  | 84929-38-4 | EC No. |  | 284-521-0 | EINECS No. | 284-521-0 |
| 1.2 | Relative identified uses of the substance or mixture and uses advised against |  |  |  |  |  |  |
|  | No additional data available. |  |  |  |  |  |  |
| 1.3 | Details of the supplier of the safety data sheet |  |  |  |  |  |  |
|  | Hughes & Co Ltd, 10 Market Street – Suite 512, Camana Bay, Grand Cayman, KY1-9006 Cayman Islands. EU Address – The Ingredient Warehouse, Station Road, Uppingham, Rutland. LE15 9TX. Email – sds@hughescompany.co.uk. |  |  |  |  |  |  |
| 1.4 | Emergency Tel. No. |  |  | + 44 (0) 1572 820267 |  |  |  |

## 02. HAZARDS IDENTIFICATION

| 2.1 | Classification of the substance or mixture |  |  |
| --- | --- | --- | --- |
|  | The Full Text for all Hazard Statements displayed in Section 16. |  |  |
| Classification (EC 1272/2008) |  |  |  |
|  | Flam. Liq. 3 – H226 Asp. Tox. 1 - H304 Skin Irrit 2 – H315 Skin Sens. 1 – H317 Aquatic Chronic 1 - H410 |  |  |
| 2.2 |  | Label Elements |  |
| Label in accordance with (EC) No 1272/2008 |  |  |  |
|  | GHS08 GHS07 GHS09 GHS02 |  |  |
| Signal Word |  |  | Danger. |
| Contains |  |  | - |
| Hazard Statements |  |  |  |
|  | H226 Flammable liquid and vapour. H304 May be fatal if swallowed and enters airways. H315 Causes skin irritation. H317 May cause an allergic skin reaction. H410 Very toxic to aquatic life with long lasting effects. |  |  |
| Precautionary Statements |  |  |  |
|  | P210 Keep away from heat/sparks/open flames/hot surfaces. No smoking. P241 Use explosion-proof electrical / ventilating / lighting equipment. P262 Do not get in eyes, on skin or on clothing. P273 Avoid release to the environment. P280 Wear protective gloves/protective clothing and eye/face protection. P301+310 IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician. P303+P361+P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. P331 Do NOT induce vomiting. P405 Store locked up. P501 Dispose of contents/container to … |  |  |
| Supplementary Precautionary Statements |  |  |  |
|  | None. |  |  |
| 2.3 |  | Other Hazards |  |
| PBT or vPvB according to Annex XIII |  |  | No additional data available. |
| Adverse physio-chemical properties |  |  | No additional data available. |
| Adverse effects on human health |  |  | No additional data available. |

##### 03. COMPOSITION/INFORMATION ON INGREDIENTS

| 3.2 | Mixtures |
| --- | --- |
|  | ~74.0% Limonene CAS-No.: 5989-27-5 EC No.: 227-813-5 Classification (EC 1272/2008) Flam. Liq. 3 – H226, Skin Irrit 2 – H315 Asp Tox. 1 – H304, Skin Sens. 1 – H317 Aquatic Acute 1 – H400, Aquatic Chronic 1 – H410 |
|  | ~18.0% Y-terpinene CAS-No 99-85-4, EC 202-794-6 Classification (EC 1272/2008); Flam. Liq. 3 – H226, Asp Tox. 1 –H304; |
|  | ~2.0% Alpha-pinene CAS-No.: 80-56-8 EC No.: 201-291-9 Classification (EC 1272/2008) Flam. Liq. 3 - H226, Skin Sens. 1 - H317, Asp. Tox. 1 - H304, Aquatic Acute 1 - H400, Aquatic Chronic 1 - H410 |
|  | ~2.0% Myrcene (7-methyl-3-methyleneocta-1,6-diene) CAS-No.: 123-35-3 EC No.:204-622-5 Classification (EC 1272/2008) Flam. Liq. 3 - H226, Skin Irrit. 2 - H315, Eye Irrit. 2 - H319, Asp. Tox. 1 - H304 |

##### 04. FIRST AID MEASURES

| 4.1 | Description of first aid measures |  |
| --- | --- | --- |
| Inhalation |  | Move to fresh air for at least 15 minutes, in case of complaints seek medical attention. |
| Ingestion |  | Ask immediately medical assistance - mouth washing with water and do not provoke vomiting. |
| Skin Contact |  | Remove contaminated clothing and wash with water and soap the contaminated part – make sure you have eliminated the contamination - in case of complaints seek medical attention. |
| Eye Contact |  | abundant eye-wash for 15 minutes with pure water, make sure you have eliminated the contamination, in case of complaints seek medical attention. |
| 4.2 | Most important symptoms and effects, both acute and delayed |  |
|  | Symptoms: inhalation: can cause slight headache - contact: can cause bloodshot eyes - can cause slight skin rash. Acute and delayed effects: no post-disorder effects are reported. |  |
| 4.3 | Indication of any immediate medical attention and special treatment needed |  |
|  | Immediate medical assistance: see point 4.1. Immediate/special treatment: see point 4.1. First aid specific means: eye wash fountain / safety shower should be available in the work area. |  |

##### 05. FIRE-FIGHTING MEASURES

| 5.1 | Extinguishing Media |
| --- | --- |
|  | Suitable extinguishing media: SMALL FIRE: use CO2, foam, dry powder - LARGE FIRE: use water spray or fog - cool containing vessels with water jet in order to prevent pressure build-up, auto-ignition or explosion. Unsuitable extinguishing media: pressurized water jet. |
| 5.2 | Special hazards arising from the product |
|  | Vapours may form explosive mixture with air - in case of fire, the following can be released: carbon monoxide (CO),carbon dioxide (CO2), smoke, soot. |
| 5.3 | Advice for firefighters |
|  | Standard procedure for chemical fires - spray extinguishing media to base of flames - use adequate protections for respiratory apparatus, avoid vapour inhalation. |

##### 06. ACCIDENTAL RELEASE MEASURES

| 6.1 | Personal precautions, protective equipment and emergency procedures |
| --- | --- |
|  | For non emergency personnel: in an emergency (i.e. unintentional important release of the product in enclosed space) respiratory protection (Gas filter A, Colour code brown: consider the maximum duration for wear) must be worn. Use insulating device for respiratory protection with an independent air supply in circumstances which are unclear - use adequate protections solvent resistant: security shoes, bodysuit, gloves and protective goggles (see point 8). For emergency responders: as per non emergency personnel. Emergency procedures: remove any ignition source and ensure adequate ventilation in working areas following accidental releases. |
| 6.2 | Environmental Precautions |
|  | Keep away from drains - keep away from surface and ground water. |
| 6.3 | Methods and material for containment and cleaning up. |
|  | Keep away from heat and use non-combustible absorbing sawdust (sand, specific binder). Refer to point 13 for the appropriate methods of waste treatment. |
| 6.4 | Reference to other sections |
|  | See point 4, 8 & 13. |

##### 07. HANDLING AND STORAGE

| 7.1 | Precautions for safe handling |
| --- | --- |
|  | Safe handling: during handling keep original container closed - avoid contact with skin and eyes – wear adequate protective gloves and eye/face protection - avoid any sources of ignition – avoid exposing to high temperature during processing - maintain adequate local and general ventilation where product is handled. Hygiene at work: do not ingest or apply to the skin as such - no smoking - remove contaminated clothing – good personal washing routines should be followed - if at risk of contamination, foods, beverages and other articles of consumption must not be stored or consumed at the work areas. |
| 7.2 | Conditions for safe storage, including any incompatibilities |
|  | Container: to be stored in stainless steel drums, preferably under inert atmosphere (nitrogen), protected from day-light take note: the container used during transportation must be considered only as a temporary container and it must not be considered in any case adequate for medium or long term warehousing. Conditions: stored in a dry, aerated place, away from any heat source and ignition source. Temperature: from 5°C and 20°C. |
| 7.3 | Specific end use(s) |
|  | Use as odour agent / flavouring agent - the information of this section are not related to the use of the product in combination with any other material or any other process altering its characteristics. |

##### 08. EXPOSURE CONTROLS/PERSONAL PROTECTION

| 8.1 | Control parameters |  |  |
| --- | --- | --- | --- |
|  |  | No additional data available. |  |
| 8.2 | Exposure controls |  |  |
| Protective Equipment |  |  |  |
| Process Conditions |  |  | Provide eyewash station. |
| Engineering Measures |  |  | Where appropriate, use closed system to transfer and process this material – if appropriate, isolate mixing rooms and other areas. Where this material is used or openly handled - maintain these areas under negative air pressure relative to the rest of the plant. |
| Respiratory Equipment |  |  | Not necessary in adequate local with general ventilation - avoid breathing vapours. |
| Hand Protection |  |  | Suitable gloves tested to EN374 (should be checked regularly) - always use with clean, dry hands. |
| Eye Protection |  |  | Protective goggles with built-in-frame tested to EN166 (should be checked regularly). |
| Other Protection |  |  | Protective work clothing solvent resistant (should be checked regularly). |
| Hygiene Measures |  |  | Good personal hygiene practices are always advisable, especially when working with chemicals / oils. |
| Personal Protection |  |  | Use personal protection according to Directive 89/686/EEC. |
| Skin Protection |  |  | Wear apron or protective clothing in case of splashes. |
| Environmental Exposure Controls |  |  | The usual precautions in handling chemicals have to be observed. |

##### 09. PHYSICAL AND CHEMICAL PROPERTIES

| 9.1 | Information on basic physical and chemical properties |  |
| --- | --- | --- |
| Appearance |  | Mobile liquid. |
| Colour |  | Orange to dark yellow. |
| Odour |  | Citrus – mandarin. |
| Relative Density |  | 0.906 - 0.923 @ 20⁰C |
| Flash Point (°C) |  | 48⁰C |
| Refractive Index |  | 1.471 to 1.476 @ 20⁰C |
| Melting Point (°C) |  | No additional data available. |
| Boiling Point (°C) |  | 170 +/- 10 @ 1026 hPa. |
| Vapour Pressure |  | 186.4 Pa @ 25⁰C. |
| Solubility in Water @20°C |  | Insoluble in water. |
| Auto-ignition temperature (°C) |  | 235⁰C. |
| 9.2 | Other information |  |
|  | No additional data available. |  |

##### 10. STABILITY AND REACTIVITY

| 10.1 |  | Reactivity |
| --- | --- | --- |
|  | Substance not reactive with water - substance not reactive if used according to storage & handling conditions & identified uses (see subsection 1.2). |  |
| 10.2 |  | Chemical stability |
|  | Substance stable if used according to storage & handling conditions & identified uses (see subsection 1.2). |  |
| 10.3 |  | Possible hazardous reactions |
|  | None if used according to storage & handling conditions & identified uses (see subsection 1.2). |  |
| 10.4 |  | Conditions to Avoid |
|  | Avoid exposure to heat. |  |
| 10.5 |  | Incompatible materials |
|  | Highly oxidizing agents. |  |
| 10.6 |  | Hazardous Decomposition Products |
|  | no known hazardous decomposition products under recommended storage & handling conditions - in case of combustion: carbon monoxide (CO), carbon dioxide (CO2). |  |

##### 11. TOXOLOGICAL INFORMATION

| 11.1 |  | Information on toxicological effects |  |
| --- | --- | --- | --- |
|  | Acute Toxicity |  | LD50: 5 g/kg [source RIFM]. |
|  | Skin corrosion / irritation |  | Hazard statement: H315 causes skin irritation. |
|  | Serious eye damage / irritation |  | Not classified as irritating to the eye [source RIFM]. |
|  | Respiratory or skin sensitisation |  | Hazard statement: H317 may cause an allergic skin reaction. |
|  | Germ Cell Mutagenicity |  | No additional data available. |
|  | Carcinogenicity |  | No additional data available. |
|  | Reproductive toxicity |  | No additional data available. |
|  | STOT-single exposure |  | No additional data available. |
|  | STOT-repeated exposure |  | No additional data available. |
|  | Aspiration hazard |  | Hazard statement: H304 may be fatal if swallowed and enters airways. |
|  | Photo-toxicity |  | No additional data available. |
|  | Other Information |  | No additional data available. |

##### 12. ECOLOGICAL INFORMATION

| 12.1 |  | Toxicity |
| --- | --- | --- |
|  | Aquatic life: hazard statement: H410 very toxic to aquatic life with long lasting effects. Algae/aquatic plants: EC50 65.9 mg/l (48h) calculated value [source RIFM]. Fish/aquatic invertebrates: No additional data. PNECs: No additional data. |  |
| 12.2 |  | Persistence & degradability |
|  | This substance to be considered as a readily biodegradable substance; based on its ready biodegradability, oil does not fulfil the criteria for persistence. |  |
| 12.3 |  | Bioaccumulation Potential |
|  | Hazard statement: H410 very toxic to aquatic life with long lasting effects. |  |
| 12.4 |  | Mobility in soil |
|  | This substance is considered as a readily biodegradable NCS. Based on the ready biodegradability of the NCS, simulation tests in surface water, sediment and soil are not required in accordance with column 2 of REACH Annex IX. |  |
| 12.5 |  | Results of PBT and vPvB Assessment |
|  | This substance is not classified as: PBT or vPvb / CMR. |  |
| 12.6 |  | Other adverse effects |
|  | No additional data available. |  |

##### 13. DISPOSAL CONSIDERATIONS

| 13.1 |  | Waste treatment methods |
| --- | --- | --- |
|  | The containers used for this product must completely empty before disposal. Dispose product and/or contaminated packaging in accordance with federal, state and local environmental control regulations - disposal through the waste water is illegal. |  |

##### 14. TRANSPORT INFORMATION

| 14.1 |  | UN number |  |
| --- | --- | --- | --- |
|  |  | UN No. Road | 1169 |
|  |  | UN No. SEA | 1169 |
|  |  | UN No. AIR | 1169 |
| 14.2 |  | UN proper shipping name |  |
|  | Extracts, aromatic, liquid. |  |  |
| 14.3 |  | Transport hazard class(es) |  |
|  | ADR/RID/ADN Class 3 ADR/RID/ADN Class 3 IMDG Class 3 ICAO Class/Division 3 Transport Labels |  |  |
| 14.4 |  | Packing group |  |
|  | ADR/RID/ADN Packing group III IMDG Packing group III ICAO Packing group III |  |  |
| 14.5 |  | Environmental hazards |  |
|  | Environmentally Hazardous Substance/Marine Pollutant. |  |  |
| 14.6 |  | Special precautions for user |  |
|  | This product contains constituents flammables & dangerous for the environment - in case of pouring out, make sure to label new package accordingly, reproducing original label with relevant symbols. |  |  |
| 14.7 |  | Transport in bulk according to Annex II of MARPOL73/78 and the IBC code |  |
|  | Tariff Number 3301192000. |  |  |

##### 15. REGULATORY INFORMATION

| 15.1 |  | Safety, health and environmental regulations/legislation specific for the substance or mixture |
| --- | --- | --- |
|  | Statutory Instruments The Chemicals (Hazard Information and Packaging for Supply) Regulations 2009 (S.I 2009 No. 716). Guidance Notes Workplace Exposure Limits EH40. CHIP for everyone HSG(108). EU Legislation Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC, including amendments. |  |
| 15.2 |  | Chemical safety assessment |
|  | No additional information available. |  |

##### 16. OTHER INFORMATION

| Hazard and/or Precautionary Statements in Full | H226 flammable liquid and vapour. H304 may be fatal if swallowed and enters airways. H315 causes skin irritation. H317 may cause an allergic skin reaction. H410 very toxic to aquatic life with long lasting effects. |
| --- | --- |
| Other Information | None. |
| Revision Date | 27/10/2015 |
| Reason for revision | New format and updated SDS. |
| Rev No/Repl, SDS Generated | 02 |

## DISCLAIMER: This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.