| Product Name |  | Rosehip Oil Refined |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | 24/04/2015 |  | Product Code | - | Rev | 01 |

| Description | Rosehip Oil Refined. |  |  |
| --- | --- | --- | --- |
| Appearance | Clear liquid yellow. |  |  |
| Odour | Characteristic. |  |  |
| INCI Name | Rosa Canina Fruit Oil |  |  |
| CAS Number | 84696-47-9 / 84603-93-0 | EC Number | 283-652-0 |

## TEST SPECIFICATION

| ANALYTICAL TEST | SPECIFICATION RANGE |
| --- | --- |
| Refractive Index @ 20°C | 1.467 – 1.480 |
| Specific Gravity @ 20°C (g/ml) | 0.911 – 0.928 |
| Free Fatty Acid (% As Oleic) | ≤ 0.5 |
| Iodine Value (mg/g) | 138.0 – 179.0 |
| Peroxide Value (Meq /Kg) | ≤ 8.0 (at time of production analysis) |
| Saponification Value (mgKOH/g) | 185.0 – 206.0 |
| Acid Value (mgKOH/g) | ≤ 1.0 |
| Fatty Acid Profile (%) |  |
| C 16:0 Palmitic Acid | 2.9 – 9.3 |
| C 18:0 Stearic Acid | 1.5 – 6.9 |
| C 18:1 Oleic Acid | 11.0 – 26.0 |
| C 18:2 Linoleic Acid | 39.0 – 65.0 |
| C 18:3 Linolenic Acid | ≤ 35.0 |
| C 20:0 Arachidic Acid | ≤ 2.0 |

## STORAGE

Store product in full, tightly closed containers in a cool dry place away from heat and sunlight.

## STABILITY

When stored for more than 24 months, quality should be checked before use.